# ORC Filter Implementation Doc

Config Width = 512bits = 64bytes

Operation Encoding

| Operation Type | Encoding Used |
| --- | --- |
| <, less than | 1 |
| <=, less than equal to | 2 |
| ==, equal to | 3 |
| <>, not equal | 4 |
| >, greater than | 5 |
| >= greater than equal to | 6 |

Configuration Bits

LSB												MSB

| 1byte | 1byte | 1byte | 1byte | 4bytes | 4bytes |
| --- | --- | --- | --- | --- | --- |
| Idx flag | Range flag | Right Range OP | Left Range OP | Right Range | Left Range |

Idx Flag(1/0): Check for whether to use the filter condition(0) or the stored index(1) to filter

Range Flag(1/0): Check for whether to use both ranges(1) or only right range(0) (NOTE: currently always using both ranges)

Right Range OP: contains operation from operation encoding table

Left Range OP: contains operation from operation encoding table

Right Range: Number to check for

Left Range: Number to check for